GEMS Product Item Requirements

Version 3a

Contents

Part 2a. Rotary Clothes Dryers (Determination 2015)	8

Testing and regulatory framework	8

Appliance details	8

Test laboratory	9

Test conditions	9

Test results	9

Part 2b: Rotary Clothes Dryers (Determination 2024)	11

Testing and regulatory framework	11

Appliance details	11

Test laboratory	12

Test conditions	12

Test results	12

Part 3a: Clothes Washing Machines (Determination 2015)	14

Testing and regulatory framework	14

Appliance details	14

Test laboratory	15

Test conditions	15

Test results	15

Part 3b: Clothes Washing Machines (Determination 2024)	18

Testing and regulatory framework	18

Appliance details	18

Test laboratory	19

Test conditions	19

Test results	19

Part 4a: Dishwashers (Determination 2015)	22

Testing and regulatory framework	22

Appliance details	22

Test laboratory	23

Test conditions	23

Test results	23

Part 4b. Dishwashers (Determination 2024)	26

Testing and regulatory framework	26

Appliance details	26

Test laboratory	27

Test conditions	27

Test results	27

Part 5a: Household Refrigerating Appliances (Determination 2012)	30

Part 5b. Household Refrigerating Appliances (Determination 2019)	31

Testing and regulatory framework	31

Appliance details	31

Test laboratory	32

Test conditions	32

Test results	32

Part 6: Air Conditioners up to 65kW – including portable air conditioners (Determination 2019)	34

Testing and regulatory framework	34

Appliance details	34

Test laboratory	35

Test conditions	36

Test results	36

Part 7: Close Control Air Conditioners (Determination 2012)	40

Testing and regulatory framework	40

Appliance details	40

Test laboratory	41

Test conditions	41

Test results	41

Part 8: Liquid-chilling Packages Using the Vapour Compression Cycle (Determination 2012)	43

Testing and regulatory framework	43

Appliance details	43

Test laboratory	44

Test conditions	44

Test results	44

Part 9: Self-ballasted Compact Fluorescent Lamps for General Lighting Services (Determination 2017)	47

Testing and regulatory framework	47

Appliance details	47

Test laboratory	48

Test conditions	48

Test results	48

Part 10: Transformers and Electronic Step-down Converters for ELV Lamps (Determination 2012)	50

Part 11a. Incandescent Lamps for General Lighting Services (Determination 2016)	51

Testing and regulatory framework	51

Appliance details	51

Test laboratory	52

Test conditions	52

Test results	52

Part 11b. Incandescent Lamps for General Lighting Services (Determination 2024)	54

Testing and regulatory framework	54

Appliance details	54

Test laboratory	55

Test conditions	55

Test results	55

Part 12: Ballasts for Fluorescent Lamps (Determination 2012)	57

Part 13: Double-capped Fluorescent Lamps (Determination 2017)	58

Testing and regulatory framework	58

Appliance details	58

Test laboratory	59

Test conditions	59

Test results	60

Part 14: Electric Water Heaters (Determination 2012)	61

Testing and regulatory framework	61

Appliance details	61

Test laboratory	62

Test conditions	62

Test results	62

Part 15. Gas Water Heaters (Determination 2017)	64

Testing and regulatory framework	64

Appliance details	64

Test laboratory	64

Test conditions	65

Test results (AS/NZS5263.1.2 method of test appendix in brackets) as applicable	65

Part 16: Television (Determination 2013)	66

Testing and regulatory framework	66

Appliance details	66

Test laboratory	67

Test conditions	67

Test results	67

Part 17: Digital Television Set-top Boxes (Determination 2012)	69

Part 18: External Power Supplies (Determination 2014)	70

Testing and regulatory framework	70

Appliance details	70

Test laboratory	71

Test conditions	71

Test results	71

Part 19: Computer Monitors (Determination 2014)	73

Testing and regulatory framework	73

Appliance details	73

Test laboratory	74

Test conditions	74

Test results	74

Part 20: Computers (Determination 2013)	76

Testing and regulatory framework	76

Appliance details	76

Test laboratory	77

Test conditions	77

Test results	77

Part 21: Three Phase Cage Induction Motors (Determination 2019)	79

Testing and regulatory framework	79

Appliance details	79

Test laboratory	80

Test conditions	80

Test results	80

Part 22: Power Transformers (Determination 2012)	81

Testing and regulatory framework	81

Appliance details	81

Test laboratory	82

Test conditions	82

Test results	82

Part 23a. Refrigerated Cabinets (2020 Determination)	83

Testing and regulatory framework	83

Appliance details	83

Test laboratory	84

Test conditions	84

Test results	84

Part 23b. Refrigerated Cabinets (Determination 2024)	86

Testing and regulatory framework	86

Appliance details	86

Test laboratory	87

Test conditions	87

Test results	87

Part 24. Swimming Pool Pump-units (Determination 2021)	89

Testing and regulatory framework	89

Appliance details	89

Test laboratory	89

Test conditions	90

Test results	90

Part 25. Air Conditioners above 65kW (Determination 2022)	92

Testing and regulatory framework	92

Appliance details	92

Test laboratory	93

Test conditions	93

Test results	94

Part 26. Commercial Ice-makers (Determination 2025)	96

Testing and regulatory framework	96

Appliance details	96

Test laboratory	97

Test conditions	97

Test results	97

For MEPS compliance	97

For labelling compliance	98

Other requirements	98

Part 27. LED Lamps (Determination 2025)	99

Testing and regulatory framework	99

Appliance details	99

Test laboratory	100

Test conditions	100

Test results	100

GEMS level requirement	100

GEMS labelling requirement	101

Other requirements	103

# Part 2a. Rotary Clothes Dryers (Determination 2015)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Rotary Clothes Dryers) Determination 2015

- Test Standard applied (including any amendment): AS/NZS2442.1:1996 + A1 to A4

- Regulatory Standard applied (including any amendment): AS/NZS2442.2:2000 + A1+A2

- Any adopted variations to the standards: Determination Schedule 1 Variation to operation of AS/NZS2442.1, Determination Schedule 2 GEMS labelling requirements – GEMS star ratings and label requirements

## Appliance details

- Appliance brand and model

- Type: vented or condensing

- Control: auto-sensing, timer or manual

- Heating technology: resistance heating or heat pump or other

- Indicate whether the appliance uses conductivity for auto-sensing

- Indicate whether the appliance is a combination washer-dryer

- Country of manufacture

- Photos of the compliance plate, energy label, water label, the front of the unit, date of manufacture

- Serial number(s) of the tested product(s)

- Rated capacity

- Comparative Energy Consumption

- Star rating

- Is condensate collected in the appliance and was it configured or modified to drain for the test

- Water connection details where applicable

- Programs recommended for a dry cotton load in the user manual

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS2442.1 Appendix F

- Details of the balance, if used

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage selected

- Nominal water temperature and pressure where applicable

- Nominal temperature of the water to prepare the wet load

## Test results

- For energy tests:

  - Name of program and any settings

  - Capacity tested

  - Nominal mass of the dry load

  - Bone dry mass of the test load

  - Initial mass of the wet test load at the start of the program and moisture content

  - Indicate how the initial mass was adjusted to achieve the target mass

  - Final mass of the dried test load at the end of the program and moisture content

  - Mass of moisture removed

  - Indicate whether interpolation or balance is used for a timer dryer

  - Measured program time (to the start of the cool down period)

  - Measured energy consumption

  - Calculated energy at 6% where interpolated

  - Tested energy performance (kWh/kg moisture removed)

  - Cool down time

- Indicate whether the appliance can achieve 6% moisture content in a single setting

- Maximum fabric test results if tested

- Efficiency calculations using test measurements

  - Projected Annual Energy Consumption

  - Star Rating Index

  - Star Rating

  - Indicate whether the appliance complies with MEPS

  - Indicate whether the appliance complies with all performance requirements

The instructions for use or user manual should be supplied on request.

Note: While not specified in the standard, data on energy and power should be recorded at 1 min intervals (or shorter) throughout the test. Where possible, total dryer mass (including load) should be recorded on a balance throughout the test. Raw data should be made available for independent review on request.

# Part 2b: Rotary Clothes Dryers (Determination 2024)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Rotary Clothes Dryers) Determination 2024

- Test Standard applied (including any amendment): AS/NZS2442.1:2021 + A1 + A2

- Regulatory Standard applied (including any amendment): AS/NZS2442.2:2021 + A1 to A3

- Any adopted variations to the standards: None

## Appliance details

- Appliance brand and model

- Type: vented or condensing

- Control: auto-sensing, timer or manual

- Heating technology: resistance heating or heat pump or other

- Indicate whether the appliance uses conductivity for auto-sensing

- Indicate whether the appliance is a combination washer-dryer

- Country of manufacture

- Photos of the compliance plate, energy label, water label, the front of the unit, date of manufacture

- Serial number(s) of the tested product(s)

- Rated capacity

- Claimed program time

- Comparative Energy Consumption

- Star rating

- Is condensate collected in the appliance and was it configured or modified to drain for the test

- Water connection details where applicable

- Programs recommended for a dry cotton load in the user manual

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS2442.1 Appendix F

- Details of the balance, if used

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage selected

- Nominal water temperature and pressure where applicable

- Nominal temperature of the water to prepare the wet load

- Whether a ring main is used to supply hot/cold water to each test station

## Test results

- For energy tests:

  - Name of program and any settings

  - Capacity tested

  - Nominal mass of the dry load

  - Bone dry mass of the test load

  - Initial mass of the wet test load at the start of the program and moisture content

  - Indicate how the initial mass was adjusted to achieve the target mass

  - Final mass of the dried test load at the end of the program and moisture content

  - Mass of moisture removed

  - Indicate whether interpolation or balance is used for a timer dryer

  - Measured program time (to the end of the cool down period)

  - Measured energy consumption

  - Calculated energy at 6% where interpolated

  - Tested energy performance (kWh/kg moisture removed)

  - Cool down time (as measured)

- Indicate whether the appliance can achieve 6% moisture content in a single setting

- Maximum fabric test results if tested

- Efficiency calculations using test measurements

  - Projected Annual Energy Consumption

  - Star Rating Index

  - Star Rating

  - Indicate whether the appliance complies with MEPS

  - Indicate whether the appliance complies with all performance requirements

The instructions for use or user manual should be supplied on request.

Note: While not specified in the standard, data on energy and power should be recorded at 1 min intervals (or shorter) throughout the test. Where possible, total dryer mass (including load) should be recorded on a balance throughout the test. Raw data should be made available for independent review on request.

# Part 3a: Clothes Washing Machines (Determination 2015)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Clothes Washing Machines) Determination 2015

- Test Standard applied (including any amendment): AS/NZS2040.1:2005 +A1+A2+A3

- Regulatory Standard applied (including any amendment): AS/NZS2040.2:2005 +A1

- Any adopted variations to the standards: Determination Schedule 1 Variation to AS/NZS 2040.2:2005 – GEMS star rating and labelling requirements, Determination Schedule 2 Make-up of test loads and numbers of swatches

## Appliance details

- Appliance brand and model

- Type: drum or non-drum (for non-drum include action e.g. agitator, impeller, nutator)

- Loading: top, front, twin tub

- Indicate whether the appliance is a combination washer-dryer

- Indicate whether the appliance has multiple drums and if so, can they be operated independently

- Country of manufacture

- Photos of the compliance plate, energy label, water label, the unit with door/lid closed and open, detergent dispenser, date of manufacture

- Serial number(s) of the tested product(s)

- Rated capacity

- Comparative Energy Consumption (warm and cold if applicable)

- Star rating

- Water connections available (cold, hot & cold)

- Program recommended for a normally soiled load in the user manual, additional programs that could qualify for energy labelling

- Any claims of water or energy consumption in the instructions for use

- Method stated in the user manual for dosing detergent (image to be supplied)

- Any variations to the standard loading procedure prescribed in the user manual (image to be supplied)

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS2040.1 Appendix A

- Details of the regulated power supply system, where applicable

- Reference machine brand and model

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage selected

- Nominal hot and cold water temperature

- Nominal hot and cold water pressure

- Whether a bleed valve is present at the point of water connection for the test machine

- Whether a ring main is used to supply hot/cold water to each test station

- Water hardness

- Detergent type and source, batch number and date

- Sodium perborate source, batch number and date OR sodium percarbonate source, batch number and date

- Anti-suds agent and source

- Swatch batch and calibration curve used

## Test results

- For energy and water consumption tests (normally measured with other performance parameters):

  - Name of program and any settings including wash temperature setting

  - Water connection mode

  - Capacity tested

  - Detergent dose (main) including dosing method and any anti-suds agent

  - Details of the bleach used (sodium perborate or sodium percarbonate) and dose

  - Table showing details of each operation including time, volume of water, water supply temperature, calculated water energy corrections, electrical energy consumed and whether the heater operated as per AS/NZS2040.1 Appendix L

  - Program time and cycle time

  - Water consumption

  - Post program energy

  - End of cycle mode power

  - Off mode power

  - Delay start mode power, if applicable

  - Wash temperature and how this was determined

  - Whether any adverse indications occurred during the test (e.g. out of balance)

- For washing performance tests – as per energy test plus:

  - Total number of swatches used

  - Soil removal and standard deviation

- For spinning performance tests – as per energy test plus:

  - Bone dry mass of the load

  - Nominal mass of the dry load

  - Mass of the load at the end of the program

  - Water extraction index

- For severity of washing action performance tests – as per energy test plus:

  - Total number of swatches used

  - Severity of washing action index

- For rinsing washing performance tests – as per energy test plus:

  - Quantity of PBIS and NaOH used

  - Volume of water used to dissolve the detergent and marker

  - Dosing method used (1 or 2)

  - Mass of retained moisture in the load (mr)

  - Rinse performance score (mg/kg of load)

  - Concentration of PBIS found in the supply water

  - Concentration of PBIS found in the sample of rinse liquor

  - Concentration of PBIS found in the sample of wash liquor (optional)

- Efficiency calculations using test measurements

  - Projected Annual Energy Consumption

  - Star Rating Index

  - Star Rating

  - Indicate whether the appliance complies with all performance requirements

The instructions for use or user manual should be supplied on request.

Note: While not specified in the standard, data on energy, power, water temperature, water volume and sump temperature should be recorded at 1 min intervals (or shorter) throughout the test. Alternatively, wash temperature during the test can be recorded with a data logger added to the load. Raw data should be made available for independent review on request.

# Part 3b: Clothes Washing Machines (Determination 2024)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Clothes Washing Machines) Determination 2024

- Test Standard applied (including any amendment): AS/NZS2040.1:2021

- Regulatory Standard applied (including any amendment): AS/NZS2040.2:2021

- Any adopted variations to the standards: Note: AS/NZS2040.1:2021 Amendment 1 has been published (sodium percarbonate as a bleach) and may be applicable for some check tests

## Appliance details

- Appliance brand and model

- Type: drum or non-drum (for non-drum include action e.g. agitator, impeller, nutator)

- Loading: top, front, twin tub

- Indicate whether the appliance is a combination washer-dryer

- Indicate whether the appliance has multiple drums and if so, can they be operated independently

- Country of manufacture

- Photos of the compliance plate, energy label, water label, the unit with door/lid closed and open, detergent dispenser, date of manufacture, MEPS level applicable

- Serial number(s) of the tested product(s)

- Rated capacity

- Comparative Energy Consumption (warm and cold if applicable)

- Star rating

- Claimed program time

- Water connections available (cold, hot & cold)

- Program recommended for a normally soiled load in the user manual, additional programs that could qualify for energy labelling

- Any claims of water or energy consumption in the instructions for use

- Method stated in the user manual for dosing detergent (image to be supplied)

- Any variations to the standard loading procedure prescribed in the user manual (image to be supplied)

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS2040.1 Appendix A

- Details of the regulated power supply system, where applicable

- Reference machine brand and model

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage selected

- Nominal hot and cold water temperature

- Nominal hot and cold water pressure

- Whether a bleed valve is present at the point of water connection for the test machine

- Whether a ring main is used to supply hot/cold water to each test station

- Water hardness

- Detergent type and source, batch number and date

- Sodium perborate source, batch number and date OR sodium percarbonate source, batch number and date

- Anti-suds agent and source

- Swatch batch and calibration curve used

## Test results

Note: Where the clothes washer has multiple compartments that can operate independently, each compartment shall be separately assessed.

- For energy and water consumption tests (normally measured with other performance parameters):

  - Name of program and any settings including wash temperature setting

  - Water connection mode

  - Capacity tested

  - Detergent dose (main) including dosing method and any anti-suds agent

  - Details of the bleach used (sodium perborate or sodium percarbonate) and dose

  - Table showing details of each operation including time, volume of water, water supply temperature, calculated water energy corrections, electrical energy consumed and whether the heater operated as per AS/NZS2040.1 Appendix L

  - Program time and cycle time

  - Water consumption

  - Post program energy

  - End of cycle mode power

  - Off mode power

  - Delay start mode power, if applicable

  - Wash temperature and how this was determined

  - Whether any adverse indications occurred during the test (e.g. out of balance)

- For washing performance tests – as per energy test plus:

  - Total number of swatches used

  - Soil removal and standard deviation

- For spinning performance tests – as per energy test plus:

  - Bone dry mass of the load

  - Nominal mass of the dry load

  - Mass of the load at the end of the program

  - Water extraction index

- For severity of washing action performance tests – as per energy test plus:

  - Total number of swatches used

  - Severity of washing action index

- For rinsing washing performance tests – as per energy test plus:

  - Quantity of PBIS and NaOH used

  - Volume of water used to dissolve the detergent and marker

  - Dosing method used (1 or 2)

  - Mass of retained moisture in the load (mr)

  - Rinse performance score (mg/kg of load)

  - Concentration of PBIS found in the supply water

  - Concentration of PBIS found in the sample of rinse liquor

  - Concentration of PBIS found in the sample of wash liquor (optional)

- Efficiency calculations using test measurements

  - Projected Annual Energy Consumption

  - Star Rating Index

  - Star Rating

  - Indicate whether the appliance complies with claimed program time validity

  - Indicate whether the appliance complies with MEPS

  - Indicate whether the appliance complies with all performance requirements

The instructions for use or user manual should be supplied on request.

Note: While not specified in the standard, data on energy, power, water temperature, water volume and sump temperature should be recorded at 1 min intervals (or shorter) throughout the test. Alternatively, wash temperature during the test can be recorded with a data logger added to the load. Raw data should be made available for independent review on request.

# Part 4a: Dishwashers (Determination 2015)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Dishwashers) Determination 2015

- Test Standard applied (including any amendment): AS/NZS2007.1: 2005

- Regulatory Standard applied (including any amendment): AS/NZS2007.2: 2005 + A1

- Any adopted variations to the standards: Determination Schedule 1 Variations to operation of AS/NZS 2007.1:2005 – alternative soiling agents and preparation methods, Determination Schedule 2 Variation to AS/NZS 2007.2:2005 – GEMS star rating and labelling requirements

## Appliance details

- Appliance brand and model

- Type: on bench, freestanding, built-in or mobile; include nominal width in mm for under bench models

- Indicate whether the appliance has multiple drawers/compartments and if so, can they be operated independently

- Country of manufacture

- Photos of the compliance plate, energy label, water label, the front of the unit with door closed and the front of the unit with door open, detergent dispenser, date of manufacture

- Serial number(s) of the tested product(s)

- Place settings (rated capacity)

- Comparative Energy Consumption (primary and supplementary, if applicable)

- Star rating

- Primary and secondary water connection modes (cold, hot, dual)

- Program recommended for a normally soiled load in the user manual, additional programs that could qualify for energy labelling

- Any claims of water or energy consumption in the instructions for use

- Whether the appliance has a water softener – where present what action was taken to disable it

- Whether the user manual instructs that the door should be opened at the end of the program

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS2007.1 Appendix A

- Details of the regulated power supply system, where applicable

- Reference machine model

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage selected

- Nominal hot and cold water temperature

- Nominal hot and cold water pressure

- Whether a bleed valve is present at the point of water connection for the test machine

- Whether a ring main is used to supply hot/cold water to each test station

- Water hardness

- Detergent type and source, batch number and date

- Rinse aid type and source, batch number and date

- Load type (AS/NZS or IEC)

## Test results

- For energy tests (normally measured with washing performance):

  - Name of program and any settings

  - Water connection mode

  - Capacity tested

  - Confirm a soiled load is used

  - Table showing details of each operation including time, volume of water, water supply temperature, calculated water energy corrections, electrical energy consumed and whether the heater operated as per AS/NZS2007.1 Table K1

  - Program time and cycle time

  - Water consumption

  - Post program energy

  - End of cycle mode power

  - Off mode power

  - Delay start mode power, if applicable

  - Include a chart of the bath temperature over the test period

- For washing performance tests:

  - Name of program – reference machine

  - Detergent dose (main and pre-wash) – reference machine

  - Name of program and any settings – test machine

  - Water connection mode – test machine

  - Capacity tested – test machine, include total number of items

  - Detergent dose (main and pre-wash) – test machine

  - Rinse aid setting – test machine

  - Reference machine total wash score (sum of all items)

  - Test machine total water score (sum of all items)

  - Washing index

  - Reference machine total water and energy consumption

- For drying performance tests:

  - Name of program and any settings – test machine

  - Water connection mode – test machine

  - Capacity tested – test machine, include total number of items

  - Note whether there was any significant difference in energy and water consumption for the drying test with a clean load (c.f. a soiled load for the washing test)

  - Details of any door opening at the end of the program and source of instructions

  - Test machine total drying score (sum of all items)

  - Drying index

- Efficiency calculations using test measurements

  - Projected Annual Energy Consumption

  - Star Rating Index

  - Star Rating

  - Indicate whether the appliance complies with all performance requirements

The instructions for use or user manual should be supplied on request.

Note: While not specified in the standard, data on energy, power, water temperature, water volume and bath temperature should be recorded at 1 min intervals (or shorter) throughout the test. Raw data should be made available for independent review on request.

# Part 4b. Dishwashers (Determination 2024)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Dishwashers) Determination 2024

- Test Standard applied (including any amendment): AS/NZS2007.1: 2021 + A1

- Regulatory Standard applied (including any amendment): AS/NZS2007.2: 2021 + A1

- Any adopted variations to the standards: A forthcoming Amendment to permit IEC Detergent E as an alternative test material may be applicable for some check tests.

## Appliance details

- Appliance brand and model

- Type: on bench, freestanding, built-in or mobile; include nominal width in mm for under bench models

- Indicate whether the appliance has multiple drawers/compartments and if so, can they be operated independently

- Country of manufacture

- Photos of the compliance plate, energy label, water label, the front of the unit with door closed and the front of the unit with door open, detergent dispenser, date of manufacture, applicable MEPS level

- Serial number(s) of the tested product(s)

- Place settings (rated capacity)

- Comparative Energy Consumption (primary and supplementary, if applicable)

- Star rating

- Claimed program time

- Primary and secondary water connection modes (cold, hot, dual)

- Program recommended for a normally soiled load in the user manual, additional programs that could qualify for energy labelling

- Any claims of water or energy consumption in the instructions for use

- Whether the appliance has a water softener – where present what action was taken to disable it

- Whether the user manual instructs that the door should be opened at the end of the program

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS2007.1 Appendix A

- Details of the regulated power supply system, where applicable

- Reference machine model

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage selected

- Nominal hot and cold water temperature

- Nominal hot and cold water pressure

- Whether a bleed valve is present at the point of water connection for the test machine

- Whether a ring main is used to supply hot/cold water to each test station

- Water hardness

- Detergent type and source, batch number and date (AS/NZS or IEC)

- Rinse aid type and source, batch number and date (AS/NZS or IEC)

- Load type (AS/NZS or IEC)

## Test results

Note: Where the dishwasher has multiple compartments that can operate independently, each compartment shall be separately assessed.

- For energy tests (normally measured with washing performance):

  - Name of program and any settings

  - Water connection mode

  - Capacity tested

  - Confirm a soiled load is used

  - Table showing details of each operation including time, volume of water, water supply temperature, calculated water energy corrections, electrical energy consumed and whether the heater operated as per AS/NZS2007.1 Table K1

  - Program time and cycle time

  - Water consumption

  - Post program energy

  - End of cycle mode power

  - Off mode power

  - Delay start mode power, if applicable

  - Include a chart of the bath temperature over the test period

- For washing performance tests:

  - Name of program – reference machine

  - Detergent dose (main and pre-wash) – reference machine

  - Name of program and any settings – test machine

  - Water connection mode – test machine

  - Capacity tested – test machine, include total number of items

  - Detergent dose (main and pre-wash) – test machine

  - Rinse aid setting – test machine

  - Reference machine total wash score (sum of all items)

  - Test machine total water score (sum of all items)

  - Washing index

  - Reference machine total water and energy consumption

- For drying performance tests:

  - Name of program and any settings – test machine

  - Water connection mode – test machine

  - Capacity tested – test machine, include total number of items

  - Note whether there was any significant difference in energy and water consumption for the drying test with a clean load (c.f. a soiled load for the washing test)

  - Details of any door opening at the end of the program and source of instructions

  - Test machine total drying score (sum of all items)

  - Drying index

- Efficiency calculations using test measurements

  - Projected Annual Energy Consumption

  - Star Rating Index

  - Star Rating

  - Indicate whether the appliance complies with claimed program time validity

  - Indicate whether the appliance complies with MEPS

  - Indicate whether the appliance complies with all performance requirements

The instructions for use or user manual should be supplied on request.

Note: While not specified in the standard, data on energy, power, water temperature, water volume and bath temperature should be recorded at 1 min intervals (or shorter) throughout the test. Raw data should be made available for independent review on request.

# Part 5a: Household Refrigerating Appliances (Determination 2012)

This Determination was superseded by Part 5b: Household Refrigerating Appliances (Determination 2019). Check testing is no longer carried out against this determination.

# Part 5b. Household Refrigerating Appliances (Determination 2019)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Household Refrigerating Appliances) Determination 2019

- Test Standard applied (including any amendment): AS/NZS IEC 62552 Parts 1 to 3

- Regulatory Standard applied (including any amendment): AS/NZS4474: 2018 + A1

- Any adopted variations to the standards: Note that IEC Amendments 1 and 2 to Parts 1 to 3 are not applied at this stage.

## Appliance details

- Appliance brand and model

- Group and GEMS class

- Country of manufacture

- Photos of the compliance plate, energy label, the compressor, the front of the unit with doors closed and the front of the unit with doors open and thermocouples in place, date of manufacture

- Serial number(s) of the tested product(s)

- Comparative Energy Consumption

- Star rating

- Declared volume (on the energy label)

- Presence or otherwise of a through the door ice dispenser and/or water dispenser

- A table listing all compartments and sub-compartments with the details set out in AS/NZS4474:2018 Section F.2.5

- Presence of any variable temperature compartment and the setting selected for each test (noting that IEC62552.3 Clause A.2.5 requires the highest energy setting)

- Whether the appliance uses a multi-speed or inverter driven compressor or multiple compressors

- Refrigerant

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS IEC 62552.1:2018

- Details of the regulated power supply system

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage selected

## Test results

- For energy tests:

  - include details as specified in AS/NZS4474:2018 Section 3

  - Include details of each test point and calculations showing any interpolation, including the method selected

  - Include confirmation that each test point complies with steady state requirements set out in AS/NZS IEC 62552.3:2018 Annex B

  - Provide details of determination of incremental energy for defrost and recovery and temperature excursions set out in AS/NZS IEC 62552.3:2018 Annex B

  - Determination of defrost interval as set out in AS/NZS IEC 62552.3:2018 Annex D

  - Determination of load processing efficiency as set out in AS/NZS IEC 62552.3:2018 Annex G (measured or deemed values)

- Where pull down tests are commissioned

  - include data as per AS/NZS4474:2018 Section 4.4

  - include a chart of the sensor temperatures over the test period that demonstrate compliance

- Where storage tests are commissioned

  - include data as per AS/NZS4474:2018 Section 4.5

- For temperature sensors and/or test packages, include details of any non-standard sensor positions, including sensors or packages that have to be moved to meet clearance requirements or any cases where mirror images of positions are used

- State the mass and material of each sensor type and number of sensors used in each compartment

- Where volume is measured, provide details of how the measurement was undertaken, the results for each compartment and any uncertainties associated with the measurement when assessing compliance with the rated values

- Details of any circumvention device detected and how this was discovered and how the energy reduction mode was estimated

- Details of any ambient controlled anti-condensation heaters (where present), including an estimate of the heater power under different operating conditions and whether this is consistent with the claimed values in the registration

- Provide details, including charts (time versus temperature), for any product that does not meet the requirements of AS/NZS4474:2018 Section 4.6 (temperature excursions during defrost)

- Provide details for any product that does not meet other requirements listed in AS/NZS4474:2018 Section 4.

- Efficiency calculations using test measurements

  - Projected Annual Energy Consumption

  - Star Rating Index

  - Star Rating

  - Indicate whether the appliance complies with MEPS

  - Indicate whether the appliance complies with all performance requirements

Note: Raw data should be made available for independent review on request.

# Part 6: Air Conditioners up to 65kW – including portable air conditioners (Determination 2019)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Air Conditioners up to 65kW) Determination 2019

- Test Standard applied (including any amendment): AS/NZS3823.1.1:2012, AS/NZS3823.1.2:2012, AS/NZS3823.1.3:2005+A1, AS/NZS3823.1.4:2012, AS/NZS3823.1.5:2015, AS/NZS3823.4.1:2014+A1, AS/NZS3823.4.2:2014+A1 as applicable

- Regulatory Standard applied (including any amendment): None

- Any adopted variations to the standards: the following variations are set out in the Determination

  - Schedule 2 Testing requirements

  - Schedule 6 Commercial based temperature bins

## Appliance details

- Appliance brand and model

- Type: cooling only, reverse cycle, heating only

- Configuration 1 air distribution: ducted, non-ducted

- Configuration 2: window/wall, single split, double/triple split, multi-split, packaged, spot cooler (single duct), portable split, portable wall hung (dual duct)

- Configuration 3 heat transfer: air, water loop, cooling tower

- Non-ducted split system indoor configuration: wall-hung, under ceiling, floor mounted, cassette, mixed (multi-split only)

- GEMS Class as per Schedule 1 of the Determination and the relevant MEPS level

- Did the supplier claim part load compliance for cooling or heating MEPS (details provided in the laboratory data sheet)

- List the number of compressors

- Indicate whether the air conditioner uses a variable output compressor

- Cooling capacity shown on the label (35°C)(T1)

- Heating capacity shown on the label (7°C)(H1)

- Heating capacity shown on the label (2°C)(H2)

- Residential energy for heating and cooling (Hot, Average and Cold climates)

- Noise sound level shown on the label (indoor/outdoor)

- Rated capacity cooling (T1)

- Rated effective power input for cooling (T1)

- Rated capacity heating (H1)

- Rated effective power input for heating (H1)

- Rated capacity heating (H2) where claimed

- Rated effective power input for heating (H2) where claimed

- Rated capacity heating (H3) where claimed

- Rated effective power input for heating (H3) where claimed

- Does the appliance use evaporator condensate or another water source to cool the condenser

- Country of manufacture

- Photos of the compliance plate, energy label, general view of the unit and any markings (including compressor rating plate if accessible)

- Serial number(s) of the tested product(s)

- Rated voltage and input phases for indoor unit

- Rated voltage and input phases for outdoor unit(s)

- Refrigerant

- Length and diameter of pipe to connect components, adjustments to refrigerant charge (where applicable)

- Details of any required locking of the inverter (as applicable)

- Indicate the Demand Response capability included: DRM1, DRM2, DRM3

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Indoor test room configuration: enthalpy, calorimeter

- Outdoor test room configuration: enthalpy, calorimeter, water loop

- Confirmation that equipment and test room complies with relevant part of AS/NZS 3823

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test voltage (where applicable)

- Was the air conditioner tested with an air filter installed?

## Test results

- For each test set out the main elements

  - Mode: heating or cooling

  - Test type: cooling capacity, heating capacity, maximum cooling

  - Reference test condition: H1, H2, H3, T1, low temperature cooling, water loop, other

  - Test period

  - Average indoor test conditions: wet and dry bulb

  - Average outdoor test conditions: wet and dry bulb

  - For single duct systems: indoor-discharge airflow and outdoor exhaust airflow rate

  - Water loop temperature (where applicable)

  - Water loop flow rate (where applicable)

  - External static pressure and air flow rate for ducted systems

  - Data sampling interval

  - Measured power factor

- Cooling tests

  - Setting selected or inverter speed

  - Measured sensible cooling capacity

  - Measured latent cooling capacity

  - Measured total cooling capacity

  - Measured effective power input

  - Measured EER cooling

- Heating capacity tests

  - Setting selected or inverter speed

  - Measured heating capacity

  - Measured effective power input

  - Measured COP heating

  - Did the unit enter a defrost cycle during the test?

  - If so, what is the average interval between defrosts?

  - Indicate whether the test result reported is steady state or transient as defined in the standard

- Maximum cooling tests

  - Setting selected or inverter speed

  - Report any obvious damage to the equipment during the test

  - Details of any motor overload protection trip within the first hour

- Non-operating power measurements

  - Average power consumption at each specified temperature condition

  - Weighted average inactive power (Pia)

- Possible tests that may be requested (depending on the product type and the test submitted in the registration system) – please indicate your capability to perform heating tests at condition H3 and single duct products to AS/NZS3823.5 in your tender submission:

  - Rated capacity at T1 cooling

  - Half capacity at T1 cooling

  - Minimum capacity at T1 cooling

  - Full capacity at low temperature cooling (low temperature condition is indoor dry bulb 27°C, indoor wet bulb 19°C, outdoor dry bulb 29°C, outdoor wet bulb 19°C)

  - Low temperature half capacity cooling

  - Low temperature min capacity cooling

  - H1 rated capacity heating

  - H1 half capacity heating

  - H1 minimum capacity heating

  - H2 extended capacity heating

  - H2 rated capacity heating

  - H2 half capacity heating

  - H2 min capacity heating

  - H3 extended capacity heating

  - H3 rated capacity heating

  - H3 half capacity heating

  - H3 min capacity heating

  - Single duct systems: Rated cooling capacity at dry bulb 35°C, wet bulb 24°C for both indoor and outdoor chambers

  - Single duct systems: Rated heating capacity at dry bulb 20°C, wet bulb 15°C for both indoor and outdoor chambers

- Seasonal efficiency calculations (using the SEER calculator)(to mirror claims) – note labs can only validate seasonal performance ratings if all of the originally submitted rating points have been included in the commissioned check test

  - Total Cooling Seasonal Performance Factor (FTCSP):

    - Residential Cold

    - Residential Mixed

    - Residential Hot & Humid

    - Commercial Cold

    - Commercial Mixed

    - Commercial Hot & Humid

  - Total Cooling Energy Consumption (with 60% CIAE) (kWh):

    - Residential Cold

    - Residential Mixed

    - Residential Hot & Humid

    - Commercial Cold

    - Commercial Mixed

    - Commercial Hot & Humid

  - Star Rating Cooling:

    - Residential Cold

    - Residential Mixed

    - Residential Hot & Humid

  - Heating Seasonal Performance Factor (HSPF):

    - Residential Cold

    - Residential Mixed

    - Residential Hot & Humid

    - Commercial Cold

    - Commercial Mixed

    - Commercial Hot & Humid

  - Total Heating Energy Consumption (with 40% CIAE) (kWh):

    - Residential Cold

    - Residential Mixed

    - Residential Hot & Humid

    - Commercial Cold

    - Commercial Mixed

    - Commercial Hot & Humid

  - Star Rating Heating:

    - Residential Cold

    - Residential Mixed

    - Residential Hot & Humid

- Efficiency calculations using test measurements

  - Annual Energy Efficiency Ratio (AEER)

  - Annual Coefficient of Performance (ACOP)

  - Compliance with MEPS for cooling and/or heating

  - Indicate whether part load MEPS compliance was indicated in the registration and how the test laboratory has replicated that for the compliance assessment

The instructions for use or user manual should be supplied on request.

Note: Raw data should be made available for independent review on request.

# Part 7: Close Control Air Conditioners (Determination 2012)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Close Control Air Conditioners) Determination 2012

- Test Standard applied (including any amendment):

  - Australian/New Zealand Standard 4965.1:2008 Performance of close control air conditioners. Part 1: Testing for rating

- Regulatory Standard applied (including any amendment):

  - Australian/New Zealand Standard 4965.2:2008 Performance of close control air conditioners.  Part 2: Minimum energy performance standard (MEPS) requirement

- Any adopted variations to the standards: Nil

## Appliance details

- Appliance brand and model

- Heat transfer: air, water loop, cooling tower

- Mounting configuration: Room/Floor/Ceiling/Other

- List the number of compressors

- Indicate whether the air conditioner uses a variable output compressor

- Rated capacity cooling

- Rated effective power input for cooling

- Rated Sensible Energy Efficiency Ratio -

- Country of manufacture

- Photos of the compliance plate, energy label, general view of the unit and any markings (including compressor rating plate if accessible)

- Serial number(s) of the tested product(s)

- Rated voltage and input phases for indoor unit

- Rated voltage and input phases for outdoor unit(s)

- Refrigerant

- Length and diameter of pipe to connect components, adjustments to refrigerant charge (where applicable)

- Details of any required locking of the inverter (as applicable)

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Indoor test room configuration: enthalpy, calorimeter

- Outdoor test room configuration: enthalpy, calorimeter, water loop

- Confirmation that equipment and test room complies with AS/NZS 4965.1:2008

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Water supply temperature (where applicable)

- Test voltage (where applicable)

- Fan speed setting

- Compressor speed setting

- Indicate method of obtaining fixed output on air conditioners with variable output capacity:

- Other relevant settings

## Test results

- For each test set out the main elements

  - Reference test condition T1, other

  - Test period

  - Average indoor test conditions: wet and dry bulb

  - Average outdoor test conditions: wet and dry bulb

  - Data sampling interval

  - Measured power factor

  - Measured airflow rate (cubic metres/second

  - Measured external static pressure (Pa):

- Cooling test

  - Setting selected or inverter speed

  - Measured sensible cooling capacity

  - Measured latent cooling capacity

  - Measured total cooling capacity

  - Measured effective power input

  - Measured EER cooling

  - Measured EERS cooling

- Non-operating power measurements

  - Off mode.

  - Passive standby mode.

  - Delay start mode.

  - Average heater power (watts) with the section containing the compressor under a dry-bulb ambient temperature of 20°C ±2 K.

  - Average heater power (watts) with the section containing the compressor under a dry-bulb ambient temperature of 7°C ±2 K.

  - Average crankcase heater power

- MEPS Compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 8: Liquid-chilling Packages Using the Vapour Compression Cycle (Determination 2012)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Liquid-chilling Packages Using the Vapour Compression Cycle) Determination 2012

- Test Standard applied (including any amendment):

  - AS/NZS 4776.1.1:2008 means Australian/New Zealand Standard 4776.1.1:2008 Liquid-chilling packages using the vapour compression cycle - Part 1.1: Method of rating and testing for performance-rating

  - AS/NZS 4776.1.2:2008 means Australian/New Zealand Standard 4776.1.2:2008 Liquid-chilling packages using the vapour compression cycle. Part 1.2: Method of rating and testing for performance-testing

- Regulatory Standard applied (including any amendment):

  - AS/NZS 4776.2:2008 means Australian/New Zealand Standard 4776.2:2008 Liquid-chilling packages using the vapour compression cycle. Part 2: Minimum energy performance standard (MEPS) and compliance requirements

- Any adopted variations to the standards: Nil

## Appliance details

- Appliance brand and model

- Appliance certification

  - AHRI

  - Eurovent

  - Other

  - None

- Condenser type

  - Air cooled

  - Water cooled

- Rated capacity cooling

- Rated effective power input for cooling

- Rated COP (MEPS assessment conditions)

- Rated IPLV (MEPS assessment conditions)

- Country of manufacture

- Photos of the compliance plate, general view of the unit and any markings (including compressor rating plate if accessible)

- Serial number(s) of the tested product(s)

- Rated voltage and input phases

- Refrigerant type

- Refrigerant charge

- Length and diameter of pipe to connect components, adjustments to refrigerant charge (where applicable)

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS 4776.1.2:2008

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Water temperature

- Test voltage (where applicable)

- Relevant settings

## Test results

- For each test set out the main elements

  - Test rig details

  - Test period

  - Data sampling interval

- Water Cooled Units

  - Evaporator

    - Entering water temperature °C

    - Leaving water temperature °C

    - Water flow rate L/s

    - Water pressure drop kPa

    - Fouling factor m2.K/kW

  - Condenser

    - Entering water temperature °C

    - Leaving water temperature °C

    - Water flow rate L/s

    - Water pressure drop kPa

    - Fouling factor m2.K/kW

- Air Cooled Units

  - Evaporator

    - Entering water temperature °C

    - Leaving water temperature °C

    - Water flow rate L/s

    - Water pressure drop kPa

    - Fouling factor m2.K/kW

  - Condenser

    - Entering dry bulb air temperature °C

    - Leaving dry bulb air temperature °C

- Performance data for all types

  - Compressor

    - Input power kW

    - Frequency Hz

    - Voltage average V

    - Average current A

  - Performance

    - Total input power kW

    - Cooling capacity kW

    - Heat balance %

    - COP

    - IPLV

- MEPS Compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 9: Self-ballasted Compact Fluorescent Lamps for General Lighting Services (Determination 2017)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a checklist and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Self-ballasted Compact Fluorescent Lamps for General Lighting Services) Determination 2017

- Test Standard applied (including any amendment):

  - Australian/New Zealand Standard 4847.1:2010 Self ballasted lamps for general lighting services, Part 1: Test methods — Energy performance

- Regulatory Standard applied (including any amendment):

  - Australian/New Zealand Standard 4847.2:2010 Self ballasted lamps for general lighting services – Part 2: Minimum Energy Performance Standards (MEPS) requirements

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance Serial Number

- Country of manufacture

- Photos of the products markings, side view, top view

- Lamp cap type

- Lamp type:

  - covered or uncovered lamp

  - reflector or non-reflector lamp

- Lamp ILCOS code

- Nameplate input voltage (V)

- Nameplate input frequency (Hz)

- Nameplate input current (A)

- Nameplate input power (W)

- Claimed luminous flux (lumens)

- Claimed efficacy (lumens/Watt)

- Claimed lifetime (hours)

- GLS equivalence claim (W)

- Claimed colour rendering Index (CRI)

- Claimed mercury content (mg)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with Australian/New Zealand Standard 4847.1:2010

- Details of the regulated power supply system, where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Warm up time

- Luminance flux measurement method:

  - Integrating sphere

  - Goniophotometer (distribution photometer)

  - Spectroradiometer

  - Colorimeter

- Regulated power supply details (if used)

## Test results

- Initial efficacy (lumens/Watt)

- Initial luminous flux/Light Output (lumens)

- Power (W)

- Lifetime (hours)

- Starting time (s)

- Run-up time (s)

- Lumen maintenance @ 2000 hours (%)

- Lumen maintenance @ 5000 hours (%)

- Premature lamp failure rate (%)

- Power factor

- Colour appearance (SDCM)

- Colour Rendering Index (CRI)

- Mercury content (mg)

- Switching withstand (cycles)

- Harmonics

- Immunity

- MEPS compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 10: Transformers and Electronic Step-down Converters for ELV Lamps (Determination 2012)

As of 3 October 2024, this determination was repealed and is no longer in force.

# Part 11a. Incandescent Lamps for General Lighting Services (Determination 2016)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a checklist and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 2016

- Test Standard applied (including any amendment):

  - Australian/New Zealand Standard AS/NZS 4934.1:2014 – Incandescent lamps for general lighting services – Part 1: Test methods – Energy performance

- Regulatory Standard applied (including any amendment):

  - Australian Standard 4934.2:2021 – Incandescent lamps for general lighting services, Part 2: Energy performance and marking requirements

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance Serial Number

- Country of manufacture

- Photos of the products markings, side view, top view

- Lamp cap type

- Lamp type:

  - covered or uncovered lamp

  - reflector or non-reflector lamp

- Lamp ILCOS code

- Lamp supply:

  - Mains Voltage

  - Low Voltage

- Nameplate input voltage (V)

- Nameplate input frequency (Hz) or DC

- Nameplate input current (A)

- Nameplate input power (W)

- Claimed luminous flux (lumens)

- Claimed efficacy (lumens/Watt)

- Claimed lifetime (hours)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with Australian/New Zealand Standard 4934.1:2014

- Details of the regulated power supply system, where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Luminance flux measurement method:

  - Integrating sphere

  - Goniophotometer (distribution photometer)

  - Spectroradiometer

  - Colorimeter

- Regulated power supply details (if used)

## Test results

- Initial luminous flux/Light Output (lumens)

- Power (W) (and assessment of maximum power)

- Efficacy (lumens/Watt)

- Average lifetime (hours)

- Median lifetime (hours)

- Lumen maintenance (%)

- MEPS compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 11b. Incandescent Lamps for General Lighting Services (Determination 2024)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a checklist and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Incandescent Lamps for General Lighting Services) Determination 2024

- Test Standard applied (including any amendment):

  - Australian/New Zealand Standard AS/NZS 4934.1:2014 – Incandescent lamps for general lighting services – Part 1: Test methods – Energy performance

- Regulatory Standard applied (including any amendment):

  - Australian Standard 4934.2:2021 – Incandescent lamps for general lighting services, Part 2: Energy performance and marking requirements

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance serial number

- Country of manufacture

- Photos of the products markings, side view, top view

- Lamp cap type

- Lamp type:

  - covered or uncovered lamp

  - reflector or non-reflector lamp

- Lamp ILCOS code

- Lamp supply:

  - Mains voltage

  - Low voltage

- Nameplate input voltage (V)

- Nameplate input frequency (Hz) or DC

- Nameplate input current (A)

- Nameplate input power (W)

- Claimed luminous flux (lumens)

- Claimed efficacy (lumens/Watt)

- Claimed lifetime (hours)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with Australian/New Zealand Standard 4934.1:2014

- Details of the regulated power supply system, where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Luminance flux measurement method:

  - Integrating sphere

  - Goniophotometer (distribution photometer)

  - Spectroradiometer

  - Colorimeter

- Regulated power supply details (if used)

## Test results

- Initial luminous flux/Light Output (lumens)

- Power (W) (and assessment of maximum power)

- Efficacy (lumens/Watt)

- Average lifetime (hours)

- Median lifetime (hours)

- Lumen maintenance (%)

- MEPS compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 12: Ballasts for Fluorescent Lamps (Determination 2012)

As of 3 October 2024, this determination was repealed and is no longer in force.

# Part 13: Double-capped Fluorescent Lamps (Determination 2017)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a checklist and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Double-capped Fluorescent Lamps) Determination 2017

- Test Standard applied (including any amendment):

  - Australian/New Zealand Standard 4782.1:2004 Double-capped fluorescent lamps—Performance specifications - Part 1: General

  - Australian/New Zealand Standard 4782.3:2014 – Double-capped fluorescent lamps – Performance specifications – Part 3: Procedure for quantitative analysis of mercury present in fluorescent lamps

- Regulatory Standard applied (including any amendment):

  - Australian/New Zealand Standard 4782.2:2004 Double-capped fluorescent lamps—Performance specifications - Part 2: Minimum Energy Performance Standard (MEPS)

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance Serial Number

- Country of manufacture

- Photos of the products markings, side view, top view

- Lamp length (mm)

- Lamp diameter (mm)

- Lamp cap type

- Lamp ILCOS code

- Lamp frequency

  - 50Hz

  - High frequency

- Nameplate input voltage (V)

- Nameplate input frequency (Hz)

- Nameplate input current (A)

- Nameplate input power (W)

- Claimed luminous flux (lumens)

- Claimed efficacy (lumens/Watt)

- Claimed lifetime (hours)

- Claimed colour rendering Index (CRI)

- Claimed mercury content (mg)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with Australian/New Zealand Standard 4782.1:2004

- Details of the regulated power supply system, where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Warm up time

- Luminance flux measurement method:

  - Integrating sphere

  - Goniophotometer (distribution photometer)

  - Spectroradiometer

  - Colorimeter

- Regulated power supply details (if used)

- Details of the reference ballast

## Test results

- Initial efficacy (lumens/Watt)

- Maintained efficacy (%)

- Initial luminous flux/Light Output (lumens)

- Power (W)

- Lumen maintenance (%)

- Colour appearance (SDCM)

- Colour Rendering Index (CRI)

- Mercury content (mg)

- MEPS compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 14: Electric Water Heaters (Determination 2012)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Electric Water Heaters) Determination 2012

- Test Standard applied (including any amendment): AS/NZS4692.1 + A1

- Regulatory Standard applied (including any amendment): AS/NZS4692.2

- Any adopted variations to the standards: None - note that AS/NZS4692.1 A2 and A3 are not permitted

## Appliance details

- Appliance brand and model

- Type: unvented/displacement (max ≥120kPa), unvented/displacement (max <120kPa), vented/displacement/cistern fed, vented/displacement/attached tank, vented/free outlet, vented/falling level, heat exchange, other

- GEMS product class and applicable MEPS level

- Main heating element (bottom): kW rating and number of elements

- Secondary heating element(s): kW rating and position

- Indicate whether the water heater has a hot temperature/pressure relief valve and rated pressure

- Indicate whether the water heater has a cold temperature/pressure relief valve and rated pressure

- Country of manufacture

- Photos of the compliance plate, general view of the unit and any markings

- Serial number(s) of the tested product(s)

- Nominal storage capacity

- Rated hot water delivery capacity (Class 1 and 2 only)

- Details of options for cold and hot connections

- Indicate whether the product uses an anode

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS4692.1 Appendix B

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test supply voltage

- Nominal water temperature and pressure

- Describe how the array of temperature sensors was fitted to the tank

## Test results

- For standing heat loss tests:

  - Number of hot water sensors used and position

  - Hot water temperature set for the test

  - Controller used (appliance own controller or external)

  - Operating time before test period commenced

  - Sampling frequency

  - Average tank temperature at the start of the test

  - Average tank temperature at the end of the test

  - Test period in hours (whole number of control cycles)

  - Energy consumed over test period

  - Mass and thermal mass of the tank and fittings in contact with hot water

  - Thermal energy of the tank and fittings at the start and end of the test

  - Thermal energy of the hot water at the start and end of the test

  - Raw heat loss

  - Heat loss for corrected standard conditions and initial and final energy

  - Slope of the heat loss regression over the test period

- For hot water delivery tests (Class 1 and Class 2):

  - Hot water temperature set for the test

  - Operating time before draw-off commenced

  - Sampling frequency

  - Maximum initial hot water temperature

  - Cut-off temperature (12K below initial maximum temperature for displacement; temperature falls to 30K above the cold water temperature for heat exchanger type)

  - Volume of water delivered up to the cut-off temperature

  - Average flow rate to the cut-off temperature

  - Energy delivered in the hot water up to the cut-off temperature relative to 20°C

  - Provide a chart of water volume versus hot water temperature delivered

- For hot water delivery tests (Class 3 – heat exchange):

  - Calculated heat storage volume and rated value

  - Hot water temperature set for the test

  - Operating time before draw-off commenced

  - Sampling frequency

  - Maximum initial hot water temperature

  - Assessment of compliance with hot water delivery >40% nominal heat storage

  - Volume of water delivered up to the cut-off temperature (30K above cold inlet)

  - Average flow rate to the cut-off temperature

  - Energy delivered in the hot water up to the cut-off temperature relative to 20˚°C

  - Provide a chart of water volume versus hot water temperature delivered

- Details of mixed hot water delivery, where measured.

- Efficiency calculations using test measurements

  - Measured heat loss value

  - Indicate whether the appliance complies with MEPS

  - Indicate whether the heater complies delivery requirements

The instructions for use or user manual should be supplied on request.

Note: Raw data should be made available for independent review on request.

# Part 15. Gas Water Heaters (Determination 2017)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Gas Water Heaters) Determination 2017

- Test Standard applied (including any amendment): AS/NZS 5263.1.2:2016

- Regulatory Standard applied (including any amendment): AS/NZS4552.2:2010

- Any adopted variations to the standards: None

## Appliance details

- Appliance brand and model

- Type: storage or instantaneous

- Comparative Energy Consumption (label)

- Nominal gas consumption

- Storage type: rated storage capacity

- Indicate whether the water heater has a hot temperature/pressure relief valve

- Instantaneous type: rated hot water delivery rate (flow rate and temperature rise)

- Country of manufacture

- Photos of the compliance plate, energy label, general view of the unit and any markings

- Serial number(s) of the tested product(s)

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS 5263.1.2

- Uncertainties of measurement for all instruments used

## Test conditions

- Ambient air temperature

- Ambient air humidity

- Test voltage (where applicable)

- Gas supply pressure

- Gas composition details and heating value

- Nominal water temperature and pressure

## Test results (AS/NZS5263.1.2 method of test appendix in brackets) as applicable

- Determined gas consumption (3.4)

- Nominal test point pressure

- Capacity of storage tank (5.102.3, ZC124)

- Thermal efficiency of storage water heaters (5.13, ZC107)

- Heat output of storage water heaters (5.102.1, ZC107)

- Maintenance gas consumption of storage water heaters (5.13.102, ZC110)

- Water temperature rise of instantaneous water heaters (5.101.4 and 5.101.5)

- Thermal efficiency of instantaneous water heaters (5.13, ZC108)

- Heat output of instantaneous water heaters (5.102.1, ZC108)

- Startup heat capacity of instantaneous water heaters (ZC108)

- Annual energy consumption (Appendix AA)

- Compliance with MEPS requirement

The instructions for use or user manual should be supplied on request.

Note: Raw data should be made available for independent review on request.

# Part 16: Television (Determination 2013)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Television) Determination 2013 (No.2)

- Test Standard applied (including any amendment): AS/NZS62087.1:2010

- Test Standard applied to any associated external power supplies (including any amendment): AS/NZS 4665.1:2005 + A1

- Regulatory Standard applied (including any amendment): AS/NZS 62087.2.2:2011 + A1 + A2

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance serial number

- Country of manufacture

- Photos of the compliance plate and energy label, front and back of unit and ports

- Screen technology type (e.g. LCD(LED), OLED etc.)

- Vertical and horizontal resolution (lines)

- Screen resolution (MPx)

- Screen dimensions - Length, Width and Diagonal (cm)

- Screen area (cm2)

- Comparative Energy Consumption (label)

- Star rating

- Available port types – WLAN (WiFi), wired LAN (RJ45), HDMI, VGA, DVI, DisplayPort, USB-C, Thunderbolt, component, S-video, etc. (some of these have mini versions)

- Nameplate input voltage

- Nameplate input frequency

- Nameplate input current

- Nameplate input power

- Recommended home picture viewing mode

- Does the product have automatic brightness control (ABC)

- Note if the product uses an external power supply (EPS)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS 62087.1

- Details of the regulated power supply system, where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Warm up time

- Port type used for testing

- Test signal generation method

- Luminance measurement method

- Regulated power supply details (if used)

## Test results

- For energy performance (PAEC and MEPS)

  - Name of home picture viewing mode

  - Average power using dynamic broadcast content in home picture viewing mode

  - Power Factor using dynamic broadcast content in home picture viewing mode

  - Details of video content

  - Details of audio content

  - Signal type used

  - Method for determination of PAEC (Method 1 or Method 2)

  - Television passive standby power

  - Television active standby power

  - Time in passive standby mode

  - Time in active standby mode

  - Passive standby power – power factor

  - Active standby power – power factor

  - 14 hour calculated standby power (method 2 only)

- For Picture luminance

  - Recommended home picture viewing mode

  - Brightest picture viewing mode

  - Recommended home picture viewing mode luminance

  - Brightest picture viewing mode luminance

  - Compliance assessment with clause 2.4 of AS/NZS 62087.2.2: 2011

- For recommended home viewing mode

  - Compliance assessment with clause 2.1 of AS/NZS 62087.2.2: 2011

- For user selection of recommended home picture viewing mode

  - Compliance assessment with clause 2.2 of AS/NZS 62087.2.2: 2011

- For implementation of picture set-up or installation menus

  - Compliance assessment with clause 2.3 of AS/NZS 62087.2.2: 2011

- For on screen warning when set in store mode

  - Compliance assessment with clause 2.5 of AS/NZS 62087.2.2: 2011

- For Labelling and MEPS compliance (PAEC)

  - Calculated PAEC

  - Calculated SRI

  - Calculated Star Rating

  - MEPS compliance assessment

  - Label validity compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 17: Digital Television Set-top Boxes (Determination 2012)

As of 3 October 2024, this determination was repealed and is no longer in force.

# Part 18: External Power Supplies (Determination 2014)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: the Greenhouse and Energy Minimum Standards (External Power Supplies) Determination 2014

- Test Standard applied (including any amendment): AS/NZS4665.1:2005 + A1

- Regulatory Standard applied (including any amendment): AS/NZS4665.2:2005 + A1

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance Serial Number

- Country of manufacture

- Product Class (1 – 4)

- International efficiency mark (I to VI)

- Output type (AC or DC)

- User selectable output voltage (Yes/No)

- Photos of the compliance plate and energy performance markings

- Nameplate input voltage (range)

- Nameplate input frequency (range)

- Nameplate input current

- Nameplate input power

- Nameplate output voltage

- Nameplate output frequency

- Nameplate output current

- Nameplate output power

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS4665.1:2005

- Details of the regulated power supply system where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage (230V and 115V if dual rated)

- Test frequency (50 Hz and 60 Hz if dual rated)

- Ambient temperature

- Humidity

- Regulated power supply details (if used)

## Test results

- Warm up period (minimum 30 min in condition 1)

- Measured data for condition 1 (100% rated output)

- Measured data for condition 2 (75% rated output)

- Measured data for condition 3 (50% rated output)

- Measured data for condition 4 (25% rated output)

- Measured data for condition 5 (no load)

- Measured data for conditions 1 to 5 to be reported:

  - Input current AC

  - Input voltage AC

  - Input power AC

  - Output current (conditions 1 to 4 only)

  - Output voltage (conditions 1 to 4 only)

  - Output power (conditions 1 to 4 only)

  - Total harmonic distortion

  - True power factor

  - Average active mode efficiency – average of conditions 1 to 4

- For MEPS compliance

  - Compliance assessment with MEPS level no load power consumption requirements

  - Compliance assessment with MEPS level average active mode efficiency requirements

- For High Efficiency compliance (where claimed)

  - Compliance assessment with HE level no load power consumption requirements (Class 1 and 2 products only)

  - Compliance assessment with HE level average active mode efficiency requirements

- For labelling compliance (Energy Mark)

  - Calculated energy performance mark level

  - Label validity compliance assessment

Note: Raw data should be made available for independent review on request. Input power must be measured using a power meter (such as a digital power analyser) that accurately integrate AC voltage and current to give active power. It is preferable that AC and DC output power also be measured using a power meter that accurately integrate AC voltage and current to give active power. Data logging of measured data for input and output is preferred.

# Part 19: Computer Monitors (Determination 2014)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Computer Monitors) Determination 2014

- Test Standard applied to monitors (including any amendment): AS/NZS 5815.1:2012

- Test Standard applied to any associated external power supplies (including any amendment): AS/NZS 4665.1:2005 + A1

- Regulatory Standard applied (including any amendment): AS/NZS 5815.2:2013 + A1

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance Serial Number

- Country of manufacture

- Photos of the compliance plate and energy label, front and back of unit and ports

- Screen technology type (e.g. LCD, OLED etc.)

- Vertical and horizontal resolution (lines)

- Screen resolution (MPx)

- Screen dimensions - Length, Width and Diagonal (cm)

- Screen area (cm2)

- Product class (GEMS)

- Comparative Energy Consumption (energy label)

- Star rating

- Available port types – WLAN (WiFi), wired LAN (RJ45), HDMI, VGA, DVI, DisplayPort, USB-C, Thunderbolt, component, S-video, etc. (some of these have mini versions)

- Nameplate input voltage

- Nameplate input frequency

- Nameplate input current

- Nameplate input power

- Does the product have automatic brightness control (ABC)

- Note if the product uses an external power supply (EPS)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS 5815.1:2012

- Confirmation that the dynamic broadcast content used complies with AS/NZS 62087.1

- Details of the regulated power supply system, where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Warm up time

- Screen resolution setting selected for testing

- Port type used for testing

- Test pattern type used (for luminance measurements)

- Automatic Brightness Control Activated? (Yes/No)

- Luminance measurement method

- Regulated power supply details (if used)

## Test results

- Monitors without ABC enabled by default measured using test pattern

  - Luminance out of the box

  - Method used to adjust the luminance if required to meet target

  - Adjusted luminance recorded

  - On mode power average (test pattern)

- Monitors with ABC enabled by default measured using test pattern

  - On mode power consumption (test pattern) high ambient lighting

  - On mode power consumption (test pattern) low ambient lighting

  - Calculated on mode power average

- Monitors assessment of MEPS (GEMS class 1 and 2) on mode power – test pattern

- Monitors without ABC enabled by default - on mode power average measured using dynamic broadcast content

- Monitors with ABC enabled by default - on mode power average measured using dynamic broadcast content

  - On mode power consumption in high ambient conditions

  - On mode power consumption in low ambient conditions

  - Calculated average on mode power consumption

- Off mode power

  - Measured off mode power

  - Compliance assessment with clause 2.4 of AS/NZS 5815.2: 2013

- Standby active mode power

  - Measured standby active mode power

  - Compliance assessment with clause 2.3 of AS/NZS 5815.2: 2013

- Labelling compliance (PAEC)

  - Measured average on mode power (using Dynamic Broadcast Content)

  - 14 hour measured standby active power

  - Calculated PAEC

  - Calculated Star Rating Index

  - Calculated Star Rating

  - Label validity compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 20: Computers (Determination 2013)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a checklist and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Computers) Determination 2013

- Test Standard applied to computers (including any amendment):

  - Australian/New Zealand Standard AS/NZS 5813.1:2012 Information technology equipment - Energy performance of computers Part 1: Methods of measurement of energy performance

- Test Standard applied to any associated external power supplies (including any amendment):

  - Australian/New Zealand Standard AS/NZS 4665.1:2005 Performance of external power supplies Part 1: Test method and energy performance mark

- Test Standard applied to any associated internal power supplies (including any amendment):

  - Australian/New Zealand Standard AS/NZS 5814.1:2012 Information technology equipment – Energy performance of internal power supplies Part 1: Methods of measurement of energy performance

- Regulatory Standard applied (including any amendment):

  - Australian/New Zealand Standard AS/NZS 5813.2:2012 Information technology equipment - Energy performance of computers Part 2: Minimum energy performance standards (MEPS) for computers

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance serial number

- Country of manufacture

- Computer Class

  - Class 1: Notebook computers and tablet computers

  - Class 2: Desktop computers

  - Class 3: Integrated desktop computers

  - Class 4: Small-scale servers

- Power supply type

  - External

  - Internal

- Compliance type

  - Non Deemed-to-comply computer

  - Deemed-to-comply computer

- Photos of the compliance plate, front, back, sides and top of unit and ports

- Nameplate input voltage

- Nameplate input frequency

- Nameplate input current

- Nameplate input power

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS/NZS 5813.1:2012

- Details of the regulated power supply system, where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Warm up time

- Regulated power supply details (if used)

## Test results

- Power Measurements (W)

  - Poff = Standby level (Off mode) power

  - Psleep = Sleep mode power

  - PidleL = Long idle mode power defined in Appendix A of AS/NZS 5813.1

  - PidleS = Short idle mode power defined in Appendix A of AS/NZS 5813.1

  - Pwork = Work mode power

- Time Values (%)

  - Toff = % of year computer is in standby (off mode)

  - Tsleep = % of year computer is in sleep mode

  - TidleL = % of year computer is in long idle mode

  - TidleS = % of year computer is in short idle mode

  - Twork = % of year computer is in work mode

- Calculated typical energy consumption (TEC)

- Calculated MEPS level (TEC MEPS)

- MEPS compliance assessment

- Idle mode power (Class 4 only)

- Standby (off mode) WOL disabled (Class 4 only)

- Standby (off mode) WOL enabled (Class 4 only)

- Internal power supply efficiency (%) (Deemed-to-comply type computer only)

- Power factor (Deemed-to-comply type computer only)

Note: Raw data should be made available for independent review on request.

# Part 21: Three Phase Cage Induction Motors (Determination 2019)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a checklist and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Three Phase Cage Induction Motors) Determination 2019

- Test Standard applied (including any amendment):

  - IEC 60034-2-1 Edition 2.0 (2014) (Method 2-1-1B – Summation of losses, additional load losses according to the method of residual loss), or,

  - IEEE 112:2004 (USA) Method B (Test Procedure for Polyphase Induction Motors and Generators), or,

  - IEEE 112:2017 (USA) Method B (Test Procedure for Polyphase Induction Motors and Generators)

- Regulatory Standard applied (including any amendment): Greenhouse and Energy Minimum Standards (Three Phase Cage Induction Motors) Determination 2019

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance serial number

- Country of manufacture

- Rated output (kW)

- Frame type

- Number of poles

- Photos of the compliance plate

- Nameplate input voltage

- Nameplate input frequency

- Nameplate input current

- Nameplate speed (RPM)

- Nameplate power factor

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with the selected test standard

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Test standard applied

- Test method applied

## Test results

- For efficiency

  - Measured efficiency at 100% load (if applicable)

  - Measured efficiency at 75% load (if applicable)

  - MEPS compliance assessment at100% load - if applicable

  - MEPS compliance assessment at 75% load - if applicable

  - High Efficiency compliance assessment at100% load – if claimed

  - High Efficiency compliance assessment at 75% load – if claimed

  - Other test parameters to be recorded will depend on the particular test method used. These may include:

    - Stator winding resistance measurements

    - Load test measurements (at rated voltage)

    - Results for no-load test at variable voltage

Note: Raw data should be made available for independent review on request.

# Part 22: Power Transformers (Determination 2012)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a checklist and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Power Transformers) Determination 2012

- Test Standard applied (including any amendment):

  - Australian Standard AS 2374.1-1997 Power transformers. Part 1: General

  - Australian Standard AS 2374.1.2-2003 Power transformers.  Part 1.2: Minimum energy performance standard (MEPS) requirements for distribution transformers

  - Australian Standard AS 2735-1984 Dry-type power transformers

- Regulatory Standard applied (including any amendment)

  - Australian Standard AS 2374.1.2-2003 Power transformers.  Part 1.2: Minimum energy performance standard (MEPS) requirements for distribution transformers

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance serial number

- Country of manufacture

- Type

  - Oil-immersed

  - Dry

- Power Supply

  - Single phase

  - Three phase

- Network System Voltage

  - 11 kV

  - 22 kV

- Power Rating (kVA)

- SWER

  - Yes

  - No

- Product Class

  - Class1	Single-phase or single wire earth return oil-immersed type transformer.

  - Class 2	Three-phase, oil-immersed type transformer.

  - Class 3	Single phase or single wire earth return, dry-type transformer where the Um is 12 kV.

  - Class 4	Single phase or single wire earth return, dry-type transformer where the Um is 24 kV.

  - Class 5	Three phase, dry-type transformer where the Um is 12 kV.

  - Class 6	Three phase, dry-type transformer where the Um is 24 kV.

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with the selected test standard

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Test standard applied

- Test method applied

## Test results

- Efficiency (%)

- MEPS compliance assessment

- High Efficiency compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 23a. Refrigerated Cabinets (2020 Determination)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Refrigerated Cabinets) Determination 2020

- Test Standard applied (including any amendment):

  - Primary Standards - ISO 23953-1: 2015 and ISO 23953-2: 2015

  - Other Applicable standards

    - EN 16901:2016

    - EN 16838:2016

    - EN 16825:2016

- Regulatory Standard applied (including any amendment): none

- Any adopted variations to the standards: As per Schedule 3 Variations to standards in the Determination.

## Appliance details

- Appliance brand and model

- Class as per Schedule 1 of Greenhouse and Energy Minimum Standards (Refrigerated Cabinets) Determination 2020

- Is the unit an integral unit or a remote unit?

- Is the unit a horizontal or a vertical cabinet type?

- Cabinet duty rating (RSC only)

- Applicable M-package temperature class(s)

- Applicable climate classes

- Total Display Area (TDA)

- Is the unit classified as low sales volume?

- Is the unit classified as oversize?

- Country of manufacture

- Photos of the compliance/marking plate, energy label, the front and sides of the unit, the doors (if applicable), the inside of the unit with test packs and thermocouples in place, the unit’s compressor marking plate (if accessible)

- Serial number(s) of the tested product(s)

- Refrigerant

- Appliance external dimensions in mm

- Star rating (if applicable)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with ISO 23953-2

- Details of any regulated power supply system

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Test standard applied

- Ambient air temperature

- Ambient air humidity

- Test Room climate Class setting

- Air flow rate

## Test results

- For MEPS compliance

  - Relevant settings during test i.e.:

    - Thermostat setting

    - Lighting settings

    - Covers

    - Door actuation scheduling

    - Other

  - Measured Total Display Area as applicable (TDA)

  - Measured Cabinet Net Volume as applicable (VN)

  - Calculated Specific Energy Consumption (SEC)

  - Annual Energy Consumption (AEC)

  - The refrigerated cabinet’s Reference Annual Energy Consumption (RAEC)

  - E24h or Total Energy Consumption (TEC) as applicable*

  - Direct Electrical Energy Consumption (DEC) – as applicable*

  - Refrigeration Electrical Energy Consumption (REC) – as applicable*

  - The adjustment factor applied (af) – as applicable

  - The M coefficient for the cabinet’s product class (M) – as applicable

  - The N coefficient for the cabinet’s product class (N) – as applicable

  - Calculated Energy Efficiency Index (EEI)

  - MEPS compliance assessment

* Actual and reference values are required as applicable

- For M-Package temperature class compliance

  - Confirmation that the product meets the requirements of one of the M-package temperature classes as listed in section 32 of The Greenhouse and Energy Minimum Standards (Refrigerated Cabinets) Determination 2020.

- For Labelling compliance (EEI)

  - Calculated EEI (see above)

  - Calculated Star Rating

  - Label validity compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 23b. Refrigerated Cabinets (Determination 2024)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Refrigerated Cabinets) Determination 2024

- Test Standard applied (including any amendment):

  - Primary Standards - ISO 23953-1: 2015 and ISO 23953-2: 2015; OR ISO 23953-1: 2023 and ISO 23953-2: 2023

  - Other Applicable standards

    - EN 16901:2016

    - EN 16838:2016

    - EN 16838:2019

    - EN 16825:2016

    - IEC 60335:2020

    - ISO 22041:2019

    - ISO 22043:2020

- Regulatory Standard applied (including any amendment): none

- Any adopted variations to the standards: As per Schedule 2 Variations to standards in the Determination

## Appliance details

- Appliance brand and model

- Class as per Schedule 1 of Greenhouse and Energy Minimum Standards (Refrigerated Cabinets) Determination 2024

- Is the unit an integral unit or a remote unit?

- Is the unit a horizontal or a vertical cabinet type?

- Cabinet duty rating (RSC only)

- Applicable M-package temperature class(s)

- Applicable climate classes

- Total Display Area (TDA)

- Is the unit classified as low sales volume?

- Is the unit classified as oversize?

- Country of manufacture

- Photos of the compliance/marking plate, energy label, the front and sides of the unit, the doors (if applicable), the inside of the unit with test packs and thermocouples in place, the unit’s compressor marking plate (if accessible)

- Serial number(s) of the tested product(s)

- Refrigerant

- Appliance external dimensions in mm

- Star rating (if applicable)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with ISO 23953-2 (state year used)

- Details of any regulated power supply system

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Test standard applied

- Ambient air temperature

- Ambient air humidity

- Test Room climate Class setting

- Air flow rate

## Test results

- For MEPS compliance

  - Relevant settings during test i.e.:

    - Thermostat setting

    - Lighting settings

    - Covers

    - Door actuation scheduling

    - Other

  - Measured Total Display Area as applicable (TDA)

  - Measured Cabinet Net Volume as applicable (VN)

  - Calculated Specific Energy Consumption (SEC)

  - Annual Energy Consumption (AEC)

  - The refrigerated cabinet’s Reference Annual Energy Consumption (RAEC)

  - E24h or Total Energy Consumption (TEC) as applicable*

  - Direct Electrical Energy Consumption (DEC) – as applicable*

  - Refrigeration Electrical Energy Consumption (REC) – as applicable*

  - The adjustment factor applied (af) – as applicable

  - The M coefficient for the cabinet’s product class (M) – as applicable

  - The N coefficient for the cabinet’s product class (N) – as applicable

  - Calculated Energy Efficiency Index (EEI)

  - MEPS compliance assessment

* Actual and reference values are required as applicable

- For M-Package temperature class compliance

  - Confirmation that the product meets the requirements of one of the M-package temperature classes as listed in section 32 of The Greenhouse and Energy Minimum Standards (Refrigerated Cabinets) Determination 2024.

- For Labelling compliance (EEI)

  - Calculated EEI (see above)

  - Calculated Star Rating

  - Label validity compliance assessment

Note: Raw data should be made available for independent review on request.

# Part 24. Swimming Pool Pump-units (Determination 2021)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Swimming Pool Pump-units) Determination 2021

- Test Standard applied: (including any amendment): sections 3 to 7 of AS 5102.1:2019 (except for those specified in clause 5.9 of AS 5102.1:2019)

- Regulatory Standard applied (including any amendment): none

- Any adopted variations to the standards: none

## Appliance details

- Appliance brand and model

- Appliance serial number

- Country of manufacture

- Pump classification (single speed, two speed, multispeed etc)

- Rated input power (kW)

- Comparative Energy Consumption (label)

- Photos of the unit its compliance plate and energy label

- Nameplate input voltage

- Nameplate input frequency

- Nameplate input current

- Nameplate input power

- Star rating

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with AS 5102.1:2019

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Ambient temperature

- Water temperature

- Regulated power supply details (if used)

## Test results

- For MEPS Compliance

  - Pump speed setting or settings

  - Measured head

  - Measured flow rate (QD) (single speed units only)

  - Measured flow rate – Low Speed (QD_low) (2 speed and multi-speed)

  - Measured flow rate – High Speed (QD_high) (2 speed and multi-speed)

  - Measured flow rate – Max Speed (QD_max) (multi-speed only)

  - Motor input power (PD) (single speed units only)

  - Motor input power - Low Speed (PD_low) (2 speed and multi-speed only)

  - Motor input power - High Speed (PD_high) (2 speed and multi-speed only)

  - Motor input power - Max Speed (PD_max) (multi-speed only)

  - Calculated Daily Run Time (DRT)

  - Calculated Baseline WEF value

  - Calculated Weighted Energy Factor (WEF)

- For Labelling compliance (PAEC)

  - Calculated PAEC

  - Calculated SRI

  - Calculated Star Rating

  - Label validity compliance assessment

  - MEPS compliance assessment

- For Sound Power Level

  - The A-weighted sound power measured in accordance with clause 5.9 of AS 5102.1-2019

- For Power factor

  - Measured power factor

Note: Raw data should be made available for independent review on request.

# Part 25. Air Conditioners above 65kW (Determination 2022)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Air Conditioners above 65kW) Determination 2022

- Test Standard applied (including any amendment):

  - Test Standard applied (including any amendment): AS/NZS3823.1.1:2012, AS/NZS3823.1.2:2012, AS/NZS3823.1.3:2005+A1, AS/NZS3823.1.4:2012, AS/NZS3823.1.5:2015, AS/NZS3823.4.1:2014+A1, AS/NZS3823.4.2:2014+A1 as applicable

- Regulatory Standard applied (including any amendment):

  - This determination and referenced sections of the Greenhouse and Energy Minimum Standards (Air Conditioners up to 65kW) Determination 2019

- Any adopted variations to the standards: the following variations are set out in the Determination

  - None

## Appliance details

- Appliance brand and model

- Country of manufacture

- Photos of the compliance plate, energy label, general view of the unit and any markings (including compressor rating plate if accessible)

- Serial number(s) of the tested product(s)

- Rated voltage and input phases for indoor unit

- Rated voltage and input phases for outdoor unit(s)

- Refrigerant

- Type:

  - cooling only

  - reverse cycle

  - heating only

- Configuration:

  - Ducted

  - Non-ducted

- GEMS Class as per Schedule 1 of the Determination and the relevant MEPS level:

  - Class 24: Air to air unitary air conditioners

  - Class 25: Air to air single-split systems

  - Class 26: Air to air single split outdoor units (not supplied or offered for supply as part of a single split system)

  - Class 27: Air to air multi-split outdoor units (whether or not supplied or offered for supply as part of a multi-split system)

- Rated capacity cooling

- Rated effective power input for cooling

- Rated capacity heating

- Rated effective power input for heating

- Details of any required locking of the inverter (as applicable)

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Test standard applied

- Ambient air temperature

- Ambient air humidity

- Test voltage (where applicable)

- Measurement method used

  - Air enthalpy

  - Balanced calorimeter

## Test results

- For each test set out the main elements

  - Mode: heating or cooling

  - Test type: cooling capacity, heating capacity

  - Reference test condition, H1 or T1, water loop

  - Indoor test room configuration: enthalpy, calorimeter

  - Outdoor test room configuration: enthalpy, calorimeter, water loop

  - Test period

  - Average indoor test conditions: wet and dry bulb

  - Average outdoor test conditions: wet and dry bulb

  - Water loop temperature (where applicable)

  - Water loop flow rate (where applicable)

  - External static pressure and air flow rate for ducted systems

  - Data sampling interval

- Cooling tests

  - Setting selected or inverter speed

  - Measured sensible cooling capacity

  - Measured latent cooling capacity

  - Measured total cooling capacity

  - Measured effective power input

  - Measured EER cooling

  - Measured power factor

- Heating tests

  - Setting selected or inverter speed

  - Measured heating capacity

  - Measured effective power input

  - Measured COP heating

  - Measured power factor

  - Did the unit enter a defrost cycle during the test?

  - If so, what is the average interval between defrosts?

  - Indicate whether the test result reported is steady state or transient as defined in the standard

- Non-operating power measurements

  - Weighted average power consumption at each specified temperature condition

  - Weighted average inactive power (Pia)

- Efficiency calculations using test measurements

  - Annual Energy Efficiency Ratio (AEER)

  - Annual Coefficient of Performance (ACOP)

  - Compliance with MEPS for cooling and/or heating

  - Indicate whether part load MEPS compliance was indicated in the registration and how the test laboratory has replicated that for the compliance assessment

The instructions for use or user manual should be supplied on request.

Note: Raw data should be made available for independent review on request.

# Part 26. Commercial Ice-makers (Determination 2025)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (Commercial Ice makers) Determination 2025

- Test Standard applied (including any amendment):

  - Primary Standard - AS/NZS 4865.1:2008;

  - Other Applicable standards

    - ISO 6369:2023 (but only in relation to product classes 1-8);

    - ANSI/ASHRAE 29-2009 in association with AHRI 810-2023 (but only if the test is conducted at 230V, 50Hz input power);

    - ANSI/ASHRAE 29-2015 in association with AHRI 810-2023 (but only if the test is conducted at 230V, 50Hz input power);

    - ANSI/ASHRAE 29-2009 in association with AHRI 810-2016 (but only if the test is conducted at 230V, 50Hz input power);

    - ANSI/ASHRAE 29-2015 in association with AHRI 810-2016 (but only if the test is conducted at 230V, 50Hz input power).

- Regulatory Standard applied (including any amendment): none

- Any adopted variations to the standards: As per Section 16 of the Determination

## Appliance details

- Appliance brand and model

- Class as per Schedule 1 of Greenhouse and Energy Minimum Standards (Commercial Ice makers) Determination 2025

- Configuration as per Schedule 1 of Greenhouse and Energy Minimum Standards (Commercial Ice makers) Determination 2025

- Cooling mode as per Schedule 1 of Greenhouse and Energy Minimum Standards (Commercial Ice makers) Determination 2025

- Production mode as per Schedule 1 of Greenhouse and Energy Minimum Standards (Commercial Ice makers) Determination 2025

- Country of manufacture

- Declared capacity (kg/24 hrs)

- Declared energy consumption rate (kWh/100kg of ice)

- Declared condenser water use rate (L/100 kg of ice)

- Declared potable water use rate (L/100 kg of ice)

- Photos of the compliance/marking plate, energy label, the front and sides of the unit, the doors (if applicable), the inside of the unit with test packs and thermocouples in place, the unit’s compressor marking plate (if accessible)

- Serial number(s) of the tested product(s) and date of manufacture (if available)

- Refrigerant

- Appliance external dimensions in mm

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with the selected test standard

- Details of any regulated power supply system

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Test standard applied

- Ambient air temperature

- Ambient air humidity

- Air flow rate (to be confirmed)

- Test supply voltage selected

- Water supply pressure

## Test results

### For MEPS compliance

- Tested capacity (kg/24 hrs)

- Applicable MEPS constant (as per Schedule 2 of Greenhouse and Energy Minimum Standards (Commercial Ice makers) Determination 2025

- applicable MEPS variable (as per Schedule 2 of Greenhouse and Energy Minimum Standards (Commercial Ice makers) Determination 2025

- calculated maximum energy consumption rate (kWh/100kg of ice)

- tested energy consumption rate (kWh/100kg of ice)

- is the tested energy consumption rate less than or equal to the calculated maximum energy consumption rate? (Yes/No)

- MEPS Compliance assessment (Yes/No)

### For labelling compliance

- Was a label which clearly displays the GEMS registration number for the product affixed to the product or otherwise immediately and clearly visible on inspection of the product? (Yes/No)

### Other requirements

- Tested energy consumption rate (kWh/100kg of ice)

  - Is the declared energy consumption rate ≥ the tested energy consumption rate? (Yes/No)

- Tested capacity (kg/24 hrs)

  - Is the declared capacity ≤ the tested capacity? (Yes/No)

- Tested potable water use rate (L/100 kg of ice)

  - Is the declared potable water use rate ≥ the tested potable water use rate? (Yes/No)

- Tested condenser water use rate (L/100 kg of ice)

  - Is the declared condenser water use rate ≥ the tested condenser water use rate? (Yes/No)

Note: Raw data should be made available for independent review on request.

# Part 27. LED Lamps (Determination 2025)

The following items shall be included in a laboratory test report commissioned for a GEMS check test: this is a check list and the format or order of items is not specified. Laboratories will be required to fill in a pro-forma check test report summarising the acquisition of the equipment and the test results, which is affixed to the front of the normal laboratory test report for each check test commissioned.

## Testing and regulatory framework

- Regulatory instrument: Greenhouse and Energy Minimum Standards (LED Lamps) Determination 2025

- Test Standard applied (including any amendment): AS/NZS 5341: 2021 – LED lamps – Test methods – Energy and functional performance.

- Regulatory Standard applied (including any amendment): Greenhouse and Energy Minimum Standards (LED Lamps) Determination 2025

- Any adopted variations to the standards: None

## Appliance details

- Lamp brand and model

- Country of manufacture

- Photos of the lamp, energy label,

- Serial number(s) of the tested product(s) if available

- On mode power range (W)

- Lamp type – directionality (Directional or Non-Directional)

- Lamp type - beam angle range (Degrees)

- Lamp type - cap type

- Lamp type – number of caps

- Lamp length (linear type lamps only) (mm)

- Rated maximum luminance (in direction of peak intensity) (cd/mm2)

- Is the lamp rated as a high luminance LED lamp (HLLL) i.e. average luminance > 30 cd/mm2? (Yes/No)

- Lamp peak wavelength emission (nm)

- Is this a colour tunable lamp (Yes/No)

- Is this lamp sold with a containing product? (Yes/No)

- What containing product is this lamp sold with?

- What form of control gear is used with this lamp

- Can the lamp operate on control gear designed for HID lamps?

- Is this lamp a connected LED lamp (CCL)? (Yes/No)

- Does the lamp have an anti-glare shield? (Yes/No)

- Is the lamp dimmable (Yes/No)

## Test laboratory

- Laboratory accreditation details including accreditation number

- Site where the product was tested

- List of each of the test types conducted, the dates of testing and the test officer

- Description of test equipment and instruments (can be an appendix)

- Confirmation that equipment and test room complies with the noted Test Standard

- Details of the regulated power supply system where applicable

- Uncertainties of measurement for all instruments used

## Test conditions

- Test voltage

- Test frequency

- Test standard applied

- Ambient air temperature

- Ambient air humidity

- Test supply voltage

- If a directional lamp, what measurement method was used for determining the efficacy factor (F)?

  - flux in cone method with an angle of 90 or 120 degrees

  - forward facing lumens (180 degree cone)

## Test results

### GEMS level requirement

- Applicable Threshold Efficacy Factor (120)

- Applicable End Loss Factor

- Applicable Correction Factor C -Basic

- Applicable Correction Factor C (Colour Tunable)

- Applicable Correction Factor C (Anti-glare)

- Applicable Correction Factor C (High Luminance)

- Applicable Correction Factor C Total

- Applicable Efficacy Factor F

- Applicable Colour Rendering Index Factor R

- Calculated minimum required luminous efficacy (MEPS level)

- Measured luminous Efficacy (lm/W)

- Measured standby power (Psb) (W)

- Does the product meet the maximum standby power requirement? (Yes/No)

- Measured networked standby power (Pnet) (W)

- Does the product meet the maximum networked standby power requirement? (Yes/No)

### GEMS labelling requirement

- Details of markings on the lamp (note if marked and if so the marked value and unit)

  - Useful luminous flux (lm)

  - Correlated colour temperature (K)

  - The rated beam angle for directional lamps only (degrees)

  - Is there sufficient space available on the lamp for 1, 2 or all of the above marking?

- Details of markings on the packaging (note if marked and if so the marked value and unit)

  - Model identifier

  - Bar code

  - Rated useful luminous flux (lm)

  - The non-directional rated incandescent or halogen power equivalence (W)

  - The rated luminous efficacy, expressed (lm/W)

  - The rated or range of rated correlated colour temperatures (k)

  - For a directional LED lamp, the rated or range of rated beam angles (degrees)

  - Cap type

  - The rated L70B50 lifetime (hours)

  - The rated on-mode power (Pon) (W – 1 dec. place)

  - The rated standby power (Psb) (W – 2 dec. place)

  - The rated networked standby power (Pnet) (W – 2 dec. place)

  - The rated or range of rated general colour rendering index, Ra, rounded to the nearest integer

  - If the CRI <80, a clear indication on the packaging that the LED lamp is intended for use in outdoor applications or industrial applications (Not applicable/Yes/No)

  - Where the LED lamp cannot be dimmed, or can be dimmed only with specific dimmers or specific wired or wireless dimming methods a warning to that effect (Not applicable/Yes/No)

  - Where the LED lamp can only be dimmed with specific dimmers or dimming methods details of where a list of compatible dimmers or dimming methods can be found (Not applicable/
Yes/No)

- Labelling Accuracy Assessment – Test Results to be provided

| Parameter | Min No. Samples Tested | Individual sample tolerance on value marked on packaging | Tolerance on average of all samples to value marked on packaging |
| --- | --- | --- | --- |
| Rated useful luminous flux (lm) | 10 | ± 10% Or +20% if a specified integer value | - 7.5% |
| The non-directional rated incandescent or halogen power equivalence (W) | 10 | -0% of the relevant value in schedule 2 of the Determination | N/A |
| The rated luminous efficacy | Not stated | N/A | -5% |
| The rated or range of rated correlated colour temperatures | 10 | ± 10% | N/A |
| For a directional LED lamp, the rated or range of rated beam angles (degrees) | Not stated | ± 25% | N/A |
| The rated L70B50 lifetime (hours) (50% of the specimens will maintain 70% starting flux) | Nil | Displayed value on packaging must be ≤ the value declared in the registration |  |
| The rated on-mode power | 10 | + 10% | + 7.5% |
| The rated standby power | 10 | N/A | +0.1W |
| The rated networked standby power | 10 | N/A | +0.1W |
| The rated or range of rated general colour rendering index | 10 | -2 | N/A |

### Other requirements

| Parameter to be assessed | Measured Value | Does the product meet the requirement in the Determination (Yes/No) |
| --- | --- | --- |
| Measured Colour Rending Index |  |  |
| Calculated Displacement Factor |  |  |
| Measured Lumen Maintenance Factor* |  |  |
| Measured survival factor (Min. 10 samples tested, 9 to survive)* |  |  |
| Measured colour consistency |  |  |
| Measured flicker performance |  |  |
| Measured stroboscopic performance |  |  |
| Measured UV and blue hazard performance |  |  |

* These tests can take several years